- Open Geospatial Consortium Inc.

Submission Date:	2017-11-25

Approval Date:   <yyyy-mm-dd>

Publication Date:   <yyyy-mm-dd>

External identifier of this OGC® document:

Internal reference number of this OGC® document:    17-089

Version: 2.1.0

Category: OGC® Implementation Specification

Editor:  Peter Baumann

- OGC® WCS 2.1 Interface Standard - Core

Copyright © 2017 Open Geospatial Consortium, Inc. All Rights Reserved.
To obtain additional rights of use, visit http://www.opengeospatial.org/legal/.

- Warning

This document is NOT an OGC Member approved international standard. This document is available on a royalty free, non-discriminatory basis. Recipients of this document are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

- Document type:	OGC Implementation Standard

Document subtype:

Document stage:	DRAFT

Document language:	English

Content	Page

i.	Preface	iii

ii.	Terms and definitions	iii

iii.	Submitting organizations	iii

iv.	Document Contributor Contact Points	iii

v.	Revision history	iii

vi.	Changes to the OpenGIS Abstract Specification	iv

vii.	Future Work	iv

1	Scope	1

2	Conformance	1

3	Normative references	1

4	Terms and definitions	2

5	Conventions	2

5.1	UML notation	2

5.2	Data dictionary tables	2

5.4	Namespace prefix conventions	3

6	WCS data model	3

7	WCS service model	3

8	WCS Core operations	3

8.1	GetCapabilities operation	3

8.2	DescribeCoverage operation	4

8.3	GetCoverage operation	6

Annex A (normative)  Abstract test suite	7

A.1	Conformance Test Class: core	7

A.1.1	GetCapabilities lists CIS 1.1 coverages	7

A.1.2	DescribeCoverage response structure for CIS 1.1 coverages	7

A.1.3	DescribeCoverage response without CIS 1.1 partitionSet	7

A.1.4	GetCoverage response structure for CIS 1.1 coverages	8

Tables	Page

Table 1	— Namespace mappings	3

Table 2	— WCS CoverageDescription components	4

- Preface

This document specifies the service core of an OGC Web Coverage Service (WCS). As such, it is based on the Coverage Implementation Schema (CIS), OWS Common, and OGC Abstract Topic 6. This WCS 2.1 extends WCS 2.0 (which establishes how to offer coverages as per CIS 1.0) in that WCS 2.1 servers may additionally offer coverages as per CIS 1.1. Therefore, WCS 2.1 is backwards compatible with WCS 2.0: any service passing the WCS 2.1 conformance tests will also pass the WCS 2.0 tests. Further, all WCS 2.x extensions remain applicable in the context of WCS 2.1.

Suggested additions, changes, and comments on this draft document are welcome and encouraged. Such suggestions may be submitted by email message or by making suggested changes in an edited copy of this document.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. The Open Geospatial Consortium Inc. shall not be held responsible for identifying any or all such patent rights.

Recipients of this document are requested to submit, with their comments, notification of any relevant patent claims or other intellectual property rights of which they may be aware that might be infringed by any implementation of the standard set forth in this document, and to provide supporting documentation.

- Terms and definitions

This document uses the standard terms defined in Subclause 5.3 of [OGC 06-121r9], which is based on the ISO/IEC Directives, Part 2, Rules for the structure and drafting of International Standards. In particular, the word “shall” (not “must”) is the verb form used to indicate a requirement to be strictly followed to conform to this standard.

- Submitting organizations

The following organizations have submitted this Interface Specification to the Open Geospatial Consortium, Inc.:

- Jacobs University Bremen

- rasdaman GmbH

- Envitia Ltd

- European Union Satellite Center

- Document Contributor Contact Points

| Name | Organization |
| --- | --- |
| Peter Baumann | Jacobs University Bremen, rasdaman GmbH |

- Revision history

| Date | Release | Author | Paragraph modified | Description |
| --- | --- | --- | --- | --- |
| 2016-0214 | 2.1.0 | Peter Baumann | all | created |
| 201-11-25 | 2.1.0 | Peter Baumann | several | Implemented change requests from voting |

- Changes to the OpenGIS Abstract Specification

The OpenGIS® Abstract Specification does not require any changes to accommodate the technical contents of this (part of this) document.

- Future Work

No modifications to this WCS Core standard are foreseen currently.

- Introduction

The OGC Web Coverage Service (WCS) supports electronic retrieval of geospatial data as "coverages" – that is, digital geospatial information representing space/time-varying phenomena, specifically: spatio-temporal regular and irregular grids, point clouds, and general meshes.

This document specifies the WCS core; every implementation of a WCS shall adhere to this standard. This standard thus defines only basic requirements. Extensions to the core will define extensions to meet additional requirements, such as the response encoding. Indeed, additional extensions are required in order to completely specify a WCS for implementation.

This WCS 2.1 standard extends WCS 2.0 in a backwards compatible manner by accommodating coverages as per Coverage Implementation Schema (CIS) 1.1 in addition to CIS 1.0 coverages as addressed by WCS 2.0.

- OGC® WCS 2.1 Interface Standard -
Core

# Scope

This document specifies how a Web Coverage Service (WCS) offers multi-dimensional (usually: spatio-temporal) coverage data for access over the Internet. It relies on and extends WCS 2.0, which operates on coverages as per Coverage Implementation Schema (CIS) 1.0, with support for coverages as per CIS 1.1.

All WCS 2.0 service extension standards remain applicable under WCS 2.1; however, in order to accommodate CIS 1.1 some of them may need to be enhanced, leading to a minor version update like with WCS 2.1 Core itself.

# Conformance

Standardization target are WCS 2.1 implementations (currently: servers).

This document establishes a single requirements class, core, of  http://www.opengis.net/spec/WCS/2.1/req/core with a single pertaining conformance class, core, with URI http://www.opengis.net/spec/WCS/2.1/conf /core. Requirements and conformance test URIs defined in this document are relative paths to be appended to http://www.opengis.net/spec/WCS/2.1/.

Annex A lists the conformance tests which shall be exercised on any software artefact claiming to implement an OGC WCS 2.1.

# Normative references

This OGC WCS Core specification consists of the present document and an XML Schema. The complete specification is identified by OGC URI http://www.opengis.net/spec/WCS/2.1, the document has OGC URI http://www.opengis.net/doc/IS/WCS/2.1.

The standard is available for download from http://www.opengeospatial.org/standards/wcs; the XML Schema is posted online at http://schemas.opengis.net/wcs/2.1 as part of the OGC schema repository.

The following normative documents contain provisions that, through reference in this text, constitute provisions of this specification. For dated references, subsequent amendments to, or revisions of, any of these publications do not apply. For undated references, the latest edition of the normative document referred to applies.

Note	In accordance with the OGC version naming system where, in a version number x.y.z a change in x denotes a major (not necessarily backwards compatible) release, y denotes a minor (backwards compatible) release, and z denotes a corrigendum, only major version numbers are indicated below as minor revisions must be backwards compatible and, hence, can be substituted. For the same reason, no document revision is fixed, but a suffix of rX indicates that any document with matching major version number is applicable.

- OGC 07-011rX, Abstract Specification Topic 6: The Coverage Type and its Subtypes, version 7 (identical to ISO 19123:2005)

OGC 06-121r9, OGC Web Service Common Specification, version 2.0

Conformance classes used: GetCapabilities operation (Clause 7)

- OGC 09-146r2, OGC® Coverage Implementation Schema, version 1.0

Conformance classes used: coverage

- OGC 09-146r6, OGC® Coverage Implementation Schema, version 1.1

Conformance classes used: coverage

- OGC 09-110r4, OGC® Web Coverage Service – Core, version 2.0

Conformance classes used: core

# Terms and definitions

(none defined here)

# Conventions

## UML notation

Unified Modeling Language (UML) static structure diagrams appearing in this specification are used as described in Subclause 5.2 of OGC Web Service Common [OGC 06-121r9]. Further, the following conventions hold:

- UML elements having a package name of “OWS Common” are those defined in the UML model of OWS Common [OGC 06-121r9].

- UML elements having a package name of GML are those defined in the UML model of GML [OGC 07-036].

- UML elements having a package name of “SWE Common” are those defined in the UML model of SWE Common [OGC 08-094].

- UML elements having a package name of CIS are those defined in the UML model of CIS 1.0 [OGC 09-146r2].

- UML elements not qualified with a package name are those defined in this Standard.

- UML data type Any is used here as an equivalent to XML’s xsd:any.

## Data dictionary tables

The UML model data dictionary is specified herein in a series of tables. The contents of the columns in these tables are described in Subclause 5.5 of [OGC 06-121r9]. The contents of these data dictionary tables are normative, including any table footnotes.

For the reader’s convenience, table rows describing inherited components are shaded.

## 5.4	Namespace prefix conventions

The following namespaces are used in this document. The prefix abbreviations used constitute conventions used here, but are not normative. The namespaces to which the prefixes refer are normative, however.

- — Namespace mappings

| Prefix | Namespace URI | Description |
| --- | --- | --- |
| xsd | http://www.w3.org/2001/XMLSchema | XML Schema |
| gml | http://www.opengis.net/gml/3.2 | GML 3.2.1 |
| gmlcov | http://www.opengis.net/gmlcov/1.0 | CIS 1.0 (aka GMLCOV 1.0) |
| cis | http://www.opengis.net/cis/1.1 | CIS 1.1 |
| wcs | http://www.opengis.net/wcs/2.1 | WCS 2.0 |

# WCS data model

A WCS server offers a – possibly empty – set of coverage objects, each of which can be either a CIS 1.0 coverage or a CIS 1.1 coverage.

# WCS service model

The WCS Core service model is identical to WCS 2.0, but extended with CIS 1.1 coverages. In particular, the requests syntax and semantics remain unchanged so that a WCS 2.1 server offers the services established in WCS Core 2.0 [OGC 09-110r4] on both CIS 1.0 [OGC 09-146r2] and CIS 1.1 [OGC 09-146r6] coverages offered.

NOTE	As per OGC Web Services, the functionality of extensions listed in the Capabilities is available on all coverages offered, both CIS 1.0 and 1.1.

Extended behavior of WCS 2.1 over WCS 2.0 is described in the following clauses.

# WCS Core operations

## GetCapabilities operation

The GetCapabilities request is identical to its definition in WCS 2.0; the response additionally reports CIS 1.1 coverages.

- /req/core/getCapabilities-cis11:
The GetCapabilities request of a WCS 2.1 server shall handle CIS 1.0 [OGC 09-146r2] and CIS 1.1 [OGC 09-146r6] coverages identically.
Dependency: http://www.opengis.net/spec/WCS/2.0/conf/core, 
http://www.opengis.net/spec/CIS/1.0/conf/coverage, http://www.opengis.net/spec/CIS/1.1/conf/coverage

## DescribeCoverage operation

The DescribeCapabilities request is identical to its definition in WCS 2.0.

The DescribeCoverage response is identical to WCS 2.0 for CIS 1.0 coverages; for CIS 1.1 coverages, the response structure is adjusted accordingly.

- /req/core/describeCoverage-cis11:
The response to a successful DescribeCoverage request on a CIS 1.1 coverage [OGC 09-146r6] shall consist of a CoverageDescriptions element as described in Figure 1 and Table 2.

Table 2 lists the structure of the CIS 1.1 enhanced subtype WCS 2.1::CoverageDescription. Depending on the coverage type requested the result will be a WCS 2.0 coverage description (for CIS 1.0 coverages) or a WCS 2.10 description (for CIS 1.1 coverages).

- — WCS CoverageDescription components 
(extended component white, all components inherited without modifications grey)

| Name | Definition | Data type | Multiplicity |
| --- | --- | --- | --- |
| coverageFunction | GML 3.2.1 coverage function to describe how range values at coverage locations can be obtained | GML:: CoverageFunction | zero or one (optional) |
| envelope | Coverage bounding box | CIS 1.0::Envelope | zero or one (optional) |
| domainSet | Domain extent description of this coverage | CIS 1.0::DomainSet or CIS 1.1::DomainSet | zero or one (optional) |
| rangeType | Range structure description of this coverage | SWE Common:: DataRecord | one (mandatory) |
| metadata | Application-specific metadata | CIS 1.0::Metadata | zero or more (optional) |
| serviceParameters | Service-specific parameters of this coverage | WCS 2.0:: ServiceParameters | one (mandatory) |

To keep its size moderate, a coverage description does not include any recursively nested coverages, nor position/value pairs. To still provide information about the domain extent, the envelope is mandatory in this case.

- /req/core/no-description-nesting:
The response to a successful DescribeCoverage request on a CIS 1.1 coverage [OGC 09-146r6] which contains a CIS 1.1::PartitionSet shall not contain a CIS 1.1:: PartitionSet and shall contain an envelope.

NOTE	The domain set description variants added in CIS 1.1 [OGC 09-146r6] allow for recursive coverage nestings through the PartitionSet component, which obviously should be copied into a coverage description (which intuitively is expected to be comparatively small in size). However, beware that the domain extent description still can be large – the domain set of a CIS 1.0::ReferenceableGridCoverage and a CIS 1.1::DisplacementAxisNest (“warped”) coverage (CIS 1.1 conformance class grid-irregular) may grow as big as the range set, and actually can exceed its size, in certain situations.

- Figure 1 — WCS CoverageDescription UML class diagram

Example	The response to a valid DescribeCoverage request for CIS 1.1 GeneralGridCoverage with id C0001 might be:

< CoverageDescriptions
    xmlns='http://www.opengis.net/wcs/2.1/gml'
    xmlns:cis11='http://www.opengis.net/cis/1.1/gml'
    xmlns:gml='http://www.opengis.net/gml/3.2'
    xmlns:swe='http://www.opengis.net/swe/2.0'
    xmlns:xsi='http://www.w3.org/2001/XMLSchema-instance'
    xsi:schemaLocation=
                       'http://www.opengis.net/swe/2.0 http://schemas.opengis.net/sweCommon/2.0/swe.xsd
                        http://www.opengis.net/wcs/2.1/gml http://schemas.opengis.net/wcs/2.1/gml/wcsAll.xsd'>
    <CoverageDescription gml:id="C0001">
        <cis11:envelope srsName=http://www.opengis.net/def/crs/EPSG/0/4326
            axisLabels="Lat Long" srsDimension="2"> 
            <cis11:axisExtent axisLabel="Lat"  uomLabel="deg" lowerBound="1" upperBound="5"/>
            <cis11:axisExtent axisLabel="Long" uomLabel="deg" lowerBound="1" upperBound="3"/>
        </cis11:envelope>
        <cis11:domainSet>
            <cis11:generalGrid srsName=http://www.opengis.net/def/crs/EPSG/0/4326
                axisLabels="Lat Long">
                <cis11:regularAxis axisLabel="Lat"  uomLabel="deg" lowerBound="1" upperBound="5"
                    resolution="1"/>
                <cis11:regularAxis axisLabel="Long" uomLabel="deg" lowerBound="1" upperBound="3"
                    resolution="1"/>
                <cis11:gridLimits srsName="http://www.opengis.net/def/crs/OGC/0/Index2D" axisLabels="i j">
                    <cis11:indexAxis axisLabel="i" lowerBound="0" upperBound="2"/>
                    <cis11:indexAxis axisLabel="j" lowerBound="0" upperBound="2"/>
                </cis11:gridLimits>
            </cis11:generalGrid>
        </cis11:domainSet>
        <cis11:rangeType>
            <swe:DataRecord>
                <swe:field name="singleBand">
                   <swe:Quantity definition="http://www.opengis.net/def/property/OGC/0/Radiance">
                       <swe:description>Panchromatic Channel</swe:description>
                       <swe:uom code="W/cm2"/>
                   </swe:Quantity>
                </swe:field>
            </swe:DataRecord>
        </cis11:rangeType>
        <ServiceParameters>
            <CoverageSubtype>GridCoverage</CoverageSubtype>
            <nativeFormat>image/tiff</nativeFormat>
        </ServiceParameters>
    </CoverageDescription>
</CoverageDescriptions>

## GetCoverage operation

The GetCoverage request is identical to its definition in WCS 2.0.

The GetCoverage response is extended over WCS 2.0 to also allow for CIS 1.1 coverages.

- /req/core/getCoverage-cis11:
The contents of the response to a successful WCS 2.1 GetCoverage request shall be a concrete subtype of either CIS10::AbstractCoverage or CIS11::AbstractCoverage.
Dependency: http://www.opengis.net/spec/WCS/2.0/conf/core, 
http://www.opengis.net/spec/CIS/1.0/conf/coverage, http://www.opengis.net/spec/CIS/1.1/conf/coverage

NOTE	Currently, no translation between coverage types is defined so implementations should not be expected to perform any (although the specification does not exclude it); however, such functionality may be defined in future, e.g., through a bespoke WCS Extension.

### Exceptions

Exceptions are identical to their definition in WCS 2.0.

# Annex A
(normative)

Abstract test suite

A WCS implementation must satisfy the following system characteristics to be conformant with this specification.

  - Conformance Test Class: core

The OGC URI identifier of this conformance class is:
http://www.opengis.net/spec/WCS/2.1/conf/core.

Tests identifiers below are relative to http://www.opengis.net/spec/WCS/2.1/.

    - GetCapabilities lists CIS 1.1 coverages

| Test id: | /conf/core/getCapabilities-cis11 |
| --- | --- |
| Test Purpose: | Requirement /req/core/getCapabilities-cis11: The GetCapabilities request of a WCS 2.1 server shall handle CIS 1.0 [OGC 09-146r2] and CIS 1.1 [OGC 09-146r6] coverages identically. Dependency: http://www.opengis.net/spec/WCS/2.0/conf/core, http://www.opengis.net/spec/CIS/1.0/conf/coverage, http://www.opengis.net/spec/CIS/1.1/conf/coverage |
| Test method: | Provide at least one CIS 1.1 coverage on server under test. Send a GetCapabilities request to this server, check that the result lists all CIS 1.1 coverages inserted. |

    - DescribeCoverage response structure for CIS 1.1 coverages

| Test id: | /conf/core/describeCoverage-cis11 |
| --- | --- |
| Test Purpose: | Requirement /req/core/describeCoverage-cis11: The response to a successful DescribeCoverage request on a CIS 1.1 coverage [OGC 09-146r6] shall consist of a CoverageDescriptions element as described in Figure 1 and Table 2. |
| Test method: | Provide at least one CIS 1.1 coverage on the server under test. Send a DescribeCoverage request on these coverages to server under test, check the result consists of a structure as described in the requirement. Test passes if all individual tests pass. |

    - DescribeCoverage response without CIS 1.1 partitionSet

| Test id: | /conf/core/describeCoverage-cis11-no-nesting |
| --- | --- |
| Test Purpose: | Requirement /req/core/no-description-nesting: The response to a successful DescribeCoverage request on a CIS 1.1 coverage [OGC 09-146r6] which contains a CIS 1.1::PartitionSet shall not contain a CIS 1.1:: PartitionSet and shall contain an envelope. |
| Test method: | Provide at least one CIS 1.1 coverage on the server under test which contains a partitionSet. Send a DescribeCoverage request on these coverages to server under test. Check that the results fulfil the requirement. Test passes if all individual tests pass. |

    - GetCoverage response structure for CIS 1.1 coverages

| Test id: | /conf/core/getCoverage-cis11 |
| --- | --- |
| Test Purpose: | Requirement /req/core/getCoverage-cis11: The contents of the response to a successful WCS 2.1 GetCoverage request shall be a concrete subtype of either CIS10::AbstractCoverage or CIS11::AbstractCoverage. Dependency: http://www.opengis.net/spec/WCS/2.0/conf/core, http://www.opengis.net/spec/CIS/1.0/conf/coverage, http://www.opengis.net/spec/CIS/1.1/conf/coverage |
| Test method: | Provide at least one CIS 1.1 coverage on the server under test. Send a GetCoverage request on these coverages to server under test, executing all WCS Core functionalities: complete download, trimming, slicing, format encoding. Check that the results of these operations are correct as per their specification in WCS 2.0. Test passes if all individual tests pass. |

-- end of ATS --