Open Geospatial Consortium	Document OGC 08-010

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AS Topic 2 |  |  |  | CR | ? |  |  |  rev |  |  |  | - |  |  Current version: |  | ISO 19111 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | AS |  | X | Imp Spec |  |  |  | Best Practices Paper |  |  |  |  |  |  | Other |  |
| Title:  |  | Topic 2 change request - Correct grid cell terms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Arliss Whiteside |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  | 2008-02-04 |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Use one of the following categories: F (Critical correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Table 40 of ISO 19111:2007 (which is copied in proposed OGC Abstract Specification Topic 2) uses the term “grid cell” differently than used in ISO 19123 (which is copied in OGC Abstract Specification Topic 6). In ISO 19123 (which is used in WCS 1.1), the corresponding term is “sample space” which does NOT form a grid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Correct Table 39 to be consistent with ISO 19123 terms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Continued discrepancy in terms between OGC Abstract Specification Topics 2 and 6 (and between ISO 19111 and 19123) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Tables 39 and 40, B.3.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |  |  |  | GML 3.1 and 3.2.1, WCS 1.1, GML 3.1.1 grid CRS profile |  |  |  |  |  |  |
| Affected: |  |  |  | Abstract specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | X | Best Practices Papers |  |  |  |  |  |  |  |  |  | Definition identifier URNs in OGC namespace |  |  |  |  |  |  |
| Supporting Doc.  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | This change request is written as changes to ISO 19111:2007, now copied in OGC 08-015. Changes are also desirable in ISO 19123:2005 (OGC Topic 6), to correctly match ISO 19111:2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Status  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disposition  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Edit Tables 39 and 40 of ISO 19111:2007 to read:

Table 1 — Defining elements of CD_ImageDatum class

| Description: An image datum defines the origin of an image coordinate reference system, and is used in a local context only. For an image datum, the anchor is usually a grid point (the centre of a pixel or the corner of a grid cell). However, the anchor may alternately be the centre of a grid cell (half way between grid points or the centres of pixels). NOTE The image datum definition applies regardless of whether or not the image is georeferenced. Georeferencing is performed through a transformation of image CRS to geodetic or projected CRS. The transformation plays no part in the image datum definition. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stereotype: Type Class attribute: Concrete Inheritance from: CD_Datum Association roles: (aggregation) datum from CD_ImagegDatum [1], association named DefiningDatum (reverse: referenceSystem to SC_ImageCRS [0..*] navigable only from SC_ImageCRS – see Table 13) Public attributes: 8 attributes (datum name, datum alias, datum identifier, datum remarks, datum anchor, datum realization epoch, datum validity and datum scope) inherited from IO_IdentifiedObject, IO_IdentifiedObjectBase and CD_Datum (see Tables 1, 2 and 33), plus: |  |  |  |  |  |
| Attribute name | UML identifier | Data type | Obligation | Maximum Occurrence | Attribute description |
| Pixel in Cell | pixelinCell | CD_PixelinCell | M | 1 | Specification of the way the image coverage grid is associated with the image coverage range (pixel) values. |

Table 2 — Defining elements of CD_PixelinCell class

| Description: Specification of the way the image coverage grid is associated with the image data coverage range (pixel) values. As specified in ISO 19123, grid coverage range (or image pixel) values are normally for the grid points at the corners of coverage grid cells. However, some image CRSs consider pixel cells that crudely approximate sample spaces as defined in ISO 19123, cells, with image pixel values for the centres of these pixel cells. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stereotype: CodeList Inheritance from: (none) Association roles: (none) Used by: CD_ImageDatum Public attributes: |  |  |  |  |  |
| Attribute name | UML identifier | Data type | Obligation | Maximum Occurrence | Attribute description |
| Cell center | cellCenter | CharacterString | C | 1 | The origin of the image coordinate system is the centre of a coverage grid cell, midway between four positions of image pixel (range) values. |
| Cell corner | cellCorner | CharacterString | C | 1 | The origin of the image coordinate system is the corner of a coverage grid cell, as specified in ISO 19123, at the position of an image pixel (range) value. |
| Condition: One and only one of the listed attributes shall be supplied. |  |  |  |  |  |

Edit Clause B.3.5 of ISO 19111:2007 to read:

- B.3.5	Image datum

An image datum relates a coordinate system to an image (grid coverage). The image datum definition applies regardless of whether or not the image is georeferenced. Georeferencing is performed through a coordinate transformation. The coordinate transformation plays no part in the image datum definition. Image datums include an implication that the coordinate system is in the (2D) image plane. They define the origin and orientation of the coordinate system within this image plane. However, 3D images are also allowed.

The image pixel grid is defined as the set of lines of constant integer coordinate values. The term “image grid” is often used in other standards to describe the concept of Image CRS. However, care has to be taken to correctly interpret this term in the context in which it is used. The term “grid cell” is often used as a substitute for the term “pixel”, but that is not correct based on ISO 19123 [6]. Each pixel is at a grid point which is the corner of (up to) four coverage grid cells.

The origin of the image coordinate system may be associated in two ways with a pixel or grid point.  In the most frequent case, the grid is associated with the pixel data in such a way that the grid lines run through the centres of the pixels, producing grid points at the centres of the pixels. The grid points and (recorded) pixels will thus have integer coordinate values. In this case, the attribute “pixel in cell” will have the value “cell corner”.

Alternatively, the origin of the image coordinate system may be defined such that the grid lines are associated with pixel corners rather than the coverage grid cell corners. In this case, the extent of each pixel value is crudely approximated by a cell that is offset from the coverage grid cells, so that the pixel cell corners lie at the centres of coverage grid cells. The grid cell corners and pixel locations will thus have non-integer coordinate values, the fractional parts always being 0,5. The attribute “pixel in cell” will now have the value “cell centre”.

This difference in perspective has no effect on the image interpretation, but is important for coordinate transformations using coordinates in this defined image.