Open Geospatial Consortium	OGC Doc 08-158

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GML 3.2.1 |  |  |  | CR | ? |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 3.2.1 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | AS |  |  | Imp Spec |  |  |  | Recom. Paper |  |  |  |  |  |  |  |  | Other |  | X |
| Title:  |  | Extend GML Coordinate reference system schemas to include ISO 19111-2 Parametric CRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Andrew Woolf (Natural Environment Research Council) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2008-09-23 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Use one of the following categories: F (Critical correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in the TC Policies and Procedures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | ISO 19111-2 has extended the scope of ISO 19111 coordinate reference systems to include ‘parametric coordinate reference systems’ often used in environmental science communities. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Add to GML implementations of ISO 19111-2 classes: SC_ParametricCRS, CS_ParametricCS, CD_ParametricDatum. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | GML will be unable to encode important coordinate reference systems used for environmental data exchange, despite that standards-based conceptual models for these systems may be described using ISO 19111-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Abstract specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Recommendation Papers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | May need to update also 12.1.2 and D2.7 describing relationship with ISO 19111 (to include relationship with ISO 19111-2), and add references to ISO 19111-2 more generally (e.g. in 12.1.3). Also will need to add definitions to ‘Terms and definitions’ from ISO 19111-2: parametric coordinate system, parametric coordinate reference system. parametric datum, spatio-parametric coordinate reference system, spatio-parametric-temporal coordinate reference system. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Additional clauses inserted, as below:

12.3.3.37 ParametricCRS

<element name="ParametricCRS" type="gml:ParametricCRSType" substitutionGroup="gml:AbstractSingleCRS"/>

<complexType name="ParametricCRSType">

<complexContent>

<extension base="gml:ParametricCRSType">

<sequence>

<element ref="gml:parametricCS"/>

<element ref="gml:parametricDatum"/>

</sequence>

</extension>

</complexContent>

</complexType>

gml:ParametricCRS is a 1D coordinate reference system where a physical or material property or function is used as the dimension. The parameter may be measured or be a function defined in other contexts, but in parametric coordinate systems it forms the coordinate system axis. Its relationship with a spatial dimension will usually be complex and non-linear. Parametric coordinate reference systems are frequently used in the environmental sciences.

EXAMPLES Pressure in meteorological applications; density (isopycnals) in oceanographic applications.

12.3.3.38 parametricCS

<element name="parametricCS" type="gml:ParametricCSPropertyType"/>

gml:parametyricCS is an association role to the parametric coordinate system used by this CRS.

12.3.3.39 parametricDatum

<element name="parametricDatum" type="gml:ParametricDatumPropertyType"/>

gml:parametricDatum is an association role to the parametric datum used by this CRS.

12.3.3.40 ParametricCRSPropertyType

<complexType name="ParametricCRSPropertyType">

<sequence minOccurs="0">

<element ref="gml:parametricCRS"/>

</sequence>

<attributeGroup ref="gml:AssociationAttributeGroup"/>

</complexType>

gml:ParametricCRSPropertyType is a property type for association roles to a parametric coordinate reference system, either referencing or containing the definition of that reference system.